Appendix 1: Technical details of the monitoring and laboratory devices.

| Parameter | Device | Manufacturer | Measurement range | Accuracy | Resolution | Calibration method |
| --- | --- | --- | --- | --- | --- | --- |
| Precipitation / Throughfall | RG3-M Tipping Bucket Rain Gauge | Onset HOBO | / | 1% at rainfall rates up to manufacturer limits | 0.2 mm per tip | factory calibrated, periodic field checks recommended |
| Precipitation / Throughfall | Plastic Throughfall Collection Tubes | Custom-made | Passive sampling component | / | / | calibration not applicable |
| Air temperature | SHT21 Digital Sensor | Sensirion | -40 - 125 °C | ±0.3 °C | 14 bit 0.01 °C | factory calibrated |
| Relative humidity | SHT21 Digital Sensor | Sensirion | 0 - 100% | ±2% | 12 bit 0.04 % | factory calibrated |
| Wind speed | Wind Speed Sensor | Davis Instruments | 0 - 89 m s-1 | ±5% of reading | 0.4 m s-1 | factory calibrated |
| Wind direction | Wind Direction Sensor | Davis Instruments | 1 - 360° | ±3° | 22.5° on compass rose, 1° in numeric display | factory calibrated |
| Global solar radiation | Global Solar Radiation Sensor | Davis Instruments | 0 – 1800 W m-2 | ±5% | 1 W m-2 | factory calibrated |
| Volumetric soil moisture content | ECH2O EC-5 Sensor | METER Group | 0 – 100 % | ±3% Volumetric soil moisture content (typical soils) | 0.001 m3 m-3 Volumetric soil moisture content in mineral soils, 0.25% in growing media | factory calibration with option for soil-specific calibration |
| Soil temperature | DS18B20 Digital Thermometer | Maxim Integrated Products, Inc. | –10 to +85 °C | ±0.5 °C | 9–12 bit | factory calibrated |
| Data acquisition and transmission | IoTmini-mForest Data Logger | Slovenian Forestry Institute (Laboratory of Electronic Devices) | / | / | / | Time synchronization and internal diagnostics |
| Data storage and visualization | eEMIS Platform | Slovenian Forestry Institute | / | / | / | No physical sampling or calibration required |
| Leaf area index | LAI2200 Plant Canopy Analyser | LI-COR, Inc. | Wavelength Range: 320-490 nm | 1.00° | / | factory calibrated |
| pH | Metrohm pH low ionic strenght electrode | Metrohm AG | 0-14 | ±0,2 | 0,01 | two point calibration (pH=4 and 7) |
| Electrical conductivity | Metrohm conductivity electrode Pt 1000 | Metrohm AG | 0,1 - 10.000 µS/cm | ±4% | 0,01 | one point calibration (1413 µS/cm) |
| Alkalinity | Metrohm 702 SM Titrino module | Metrohm AG | 0 - 10.0000 µequ/L | ±5% | 0,1 | / |
| Anions (NO3-, NO2-, Cl-, PO43-, SO42-) | Metrohm ion chromatography modular system with Metrosep A SUPP 5 – 150/4.0 column | Metrohm AG | 0,02 – 1.000 mg/L (for all ions) | ±10% | 0,001 | 10 points calibration |
| Cations (NH4+, Na+, K+, Mn2+, Ca2+, Mg2+) | Metrohm ion chromatography modular system with Metrosep C 4 – 150/4.0 column | Metrohm AG | 0,05 – 1.000 mg/L (for all ions) | ±10% | 0,001 | 10 points calibration |
| Total nitrogen and DOC | Shimadzu TOC-L CPH + TNM-L | Shimadzu Corporation | 0,1 – 10.000 mg/L for C 0,02 – 10.000 mg/L for N | ±10% | 0,0001 | 4 points calibration |
| Water temperature | HOBO U24 | Onset HOBO | ˗2 °C – 36 °C | ±0.1 °C | 0.01 °C | factory calibrated |
| Water electrical conductivity | HOBO U24 | Onset HOBO | 0 µS/cm – 10,000 µS/cm | 3 % of range | 1 µS/cm | factory calibrated |
| Pressure | HOBO U20 | Onset HOBO | 0 kPa –145 kPa (4 m) | ±0.3 % of range | 0.014 kPa | factory calibrated |
| Discharge | FlowTracker2 | SonTek | water velocities ±0.001 – 4.0 m/s | ± 1% of range | 0.0001 m/s | factory calibrated |
| Discharge | RiverSurveyor M9 | SonTek | water velocities up to 20 m/s | ± 0.25% of range | 0.001 m/s | factory calibrated |
| Drips | Stalagmate | Driptych | / | / | 0.1 mL | factory calibrated |