File S2: Initial Passive Sampler Comparison

Purpose

To identify the most suitable passive sampler for comparison with spider web collection efficiency, we tested three conventional passive samplers under field conditions. The first passive spore trap consisted of a microscope glass slide, coated with petroleum jelly and mounted on a 1.5 m pole with a rain-protecting roof, the second passive spore trap consisted of a Ø 9 cm petri dish coated with petroleum jelly and set on a 0.5 m tall table, and the third passive spore trap consisted of a Ø 15 cm filter paper (MN 617, Macherey-Nagel, Düren, Germany) in a Ø 19 cm glass petri dish and set on a 0.5 m tall table.

Methods

Each sampler type was deployed in duplicate for 24 hours (daily) and 7 days (weekly) in the forest study area. Upon retrieval, samples were processed for DNA extraction. Petroleum-jelly was scrapped from microscope slides or petri dish using disposable scalpel and transferred into a 2 mL or 15mL tube, respectively. One quarter of filter paper was transferred into a 15mL tube. Mag-Bind Plant DNA DS Kit Protocol configured for the KingFisher Apex was followed for DNA extraction. DNA samples were analysed by qPCR using the FungiQuant (FQ) assay for total fungal DNA and the Hfrax assay for Hymenoscyphus fraxineus DNA. Spider webs (orb and sheet) were collected in parallel for comparison.

Results

Table: Passive sampler performance compared to spider webs. "Neg" indicates no amplification detected. Values are averages of three technical replicates per sample.

| Exposure | Sampler Type | FQ (Cq) | Hfrax (Cq) |
| --- | --- | --- | --- |
| Daily | Orb web | 22.46 | 33.72 |
|  | Microscope slide | 30.31 | Neg |
|  | Petri dish | 23.81 | 33.16 |
|  | Filter paper | 27.69 | 33.48 |
| Weekly | Sheet web | 21.17 | 30.81 |
|  | Microscope slide | 28.67 | Neg |
|  | Petri dish | 20.81 | 31.88 |
|  | Filter paper | 23.15 | 30.04 |

Raw data supporting this comparison are available on the https://zenodo.org/ repository under DOI: 10.5281/zenodo.17628061.

Interpretation

The petri dish covered with petroleum jelly collected the highest amount of DNA (lowest FQ Cq), but filter paper yielded comparable H. fraxineus DNA and was easier to process. The filter paper is practical for further manipulation, including sub-sampling, storage and DNA extraction and can be used for filtration of the remaining rainwater collected in the petri dish, thereby improving sensitivity and the probability of detecting the target organism. Filter paper was also demonstrated to outperform traps based on vaseline or paraffin in diversity of collected of fungal DNA (Aguayo et al., 2018). Thus, passive sampling on filter paper was selected for subsequent comparison with spider webs. Microscope slides performed poorly for both total fungi and H. fraxineus DNA, likely due to sampler setup (height position and roof for rain protection).

Aguayo, J., Fourrier-Jeandel, C., Husson, C., & Ioos, R. (2018). Assessment of Passive Traps Combined with High-Throughput Sequencing To Study Airborne Fungal Communities. Applied and Environmental Microbiology, 84(11), e02637-17. https://doi.org/10.1128/AEM.02637-17